MIDAS Usage with the PiBeta setup

S. Ritt, July 97

Introduction

The MIDAS data acquisition system reads trigger data from CAMAC, writes it to disk/tape and presents histograms and N-tuples to PAW. Each of these tasks is performed by one process: Frontend, Logger and Analyzer. All parameters of the system are stored in an "online database". They can be edited with the ODBEdit program.

Starting DAQ

First, start the data acquisition by double clicking the "Logger" and "Analyzer" icons in the yellow folder labeled "midas" on the Windows NT desktop.  If these processes are already running (icon in the Windows task bar at the bottom of the screen is present), don't start them again. Then start the two frontend processes down in the area. The trigger frontend can simple be rebooted, on the slow control frontend the icon "SC frontend" should be double clicked. After all clients have been started, they can be checkes with the "scl" (show clients) command in ODBEdit.

Controlling DAQ

Start ODBedit. This program is a simple command interface to the MIDAS system. Following commands are used most often:

start			to start a run

stop			to stop a run

histo analyzer <id>	to clear histos/n-tuples

cd <dir>			to go to a subdirectory

cd ..			go one directory up

dir			to list a directory

set <key> <value>	to set a value in the ODB

If a directory or key contains blanks, embrace them with quotation marks like

set "/runinfo/run number" 123

Following values are of major interest:

- /runinfo/run number

the current run number, gets incremented by the start command

- /logger/channels/0/state

set this value to "on" or "off" to enable logging to disk

- /logger/channels/1/state

set this value to "on" or "off" to enable logging to tape

- /logger/channels/0/statistics

contains the number of logged events

- /equipment/trigger/variables

contains the last trigger event

- /equipment/trigger/settings

contains CAMAC parameters

- /analyzer/parameters

contains analyzer parameters

Viewing histos

Start PAW. The \online\pawlogon.kumac file attaches automatically to the online data with the "hi/gl onln" command.

Now all standard PAW commands can be used. Follwing histograms are defined:

1000-		ADC histos

2000-		TDC histos

3000-		ADC sum histos

4000-		wire chamber histos

These histograms are saved into hisxxxx.rz (where xxxx is the run number) in the e:\ directory at the end of a run. They can be loaded later into PAW with

PAW> hi/file 1 e:\run00123.rz

Paw then switches to the internal //LUN1 directory, which contains the

histograms from run 123. To switch back to the online histos, enter

PAW> cd //onln

Three online N-tuples are defined:

1		Trigger event

2		Scaler event

4		Temperature event

These N-tuples contains the last 10000 events. After 10000 events, the first events gets overwritten (ring-buffer). All N-tuple operations like

PAW> nt/pl 1.cadc0%cadc1 thit0>100

can be used.

To make MIDAS controllable from inside PAW, following macros were defined:

exec start	to start a run

exec stop	to stop a run

exec clear	to clear all histos

exec clear 1	to clear N-tuple 1

exec print1	to prepare for printing

exec print2	to print everything which was displayed between print1 and print2

exec cleanup	Reset the DAQ system

Troubleshooting

If the system hangs, never reboot the computer! Instead, enter the PAW command

exec cleanup

This command stops the all programs. Then restart all processes as described above. Note that the run number and all run parameters are reset to default values. Please check them and modify if necessary with ODBEdit.

--------------oOo---------------