# Midas CAMAC Standard (MCStd)

Highlight : Added functions or CHANGES

Highlight : SR

Highlight : PAA

Highlight :

MCStd syntax:		CAMab_cde

a: size		: 16, 24, 8 allowed, but not mandatory.

b: action	: I=read, O=write, C=command no data, E=external address, or combination

c: function	: R=Repeat, S=Scan.

d: target	: N=Station, A=subaddress

e: status	: Q=return X and Q

Input:

cam16i (c,n,a,f,&d )				16 bit input

cam24i (c,n,a,f,&d )				32 bit input

cam16i_q (c,n,a,f,&d )				16 bit input with X and Q

cam24i_q (c,n,a,f,&d )				32 bit input with X and Q

cam16i_r (c,n,a,f,&d,r) 				16 bit input repeat until r

cam24i_r (c,n,a,f,&d,r) 				32 bit input repeat until r

cam16i_rq (c,n,a,f,&d,r) 			16 bit Input repeat until noQ or max r

cam24i_rq (c,n,a,f,&d,r) 			32 bit input repeat until noQ or max r

cam16i_saq (c,n,a,f,&d,r) 			16 bit input scan a until noQ or max r

cam24i_saq (c,n,a,f,&d,r) 			32 bit input scan a until noQ or max r

cam16i_snq (c,n,a,f,&d,r) 			16 bit input scan n until noQ or max r

cam24i_snq (c,n,a,f,&d,r) 			32 bit input scan n until noQ or max r

cami (c,n,a,f,&d )				16 bit input => cam16i

Output:

cam16o (c,n,a,f,d)				16 bit output

cam24o (c,n,a,f,d )				32 bit output

cam16o_q (c,n,a,f,d,&x,&q)			16 bit output with X and Q

cam24o_q (c,n,a,f,d,&x,&q )			32 bit output with X and Q

camo (c,n,a,f,d)					16 bit output => cam16o

Commands:

camc_chk (b,c,n,a)				cmd check bcna hardware validity

camc (c,n,a,f)					cmd16 no data

camc_q (c,n,a,f,&x,&q)				cmd16 no data with X and Q

camc_sa (c,n,a,f,r)				cmd16 scan A r time no data

camc_sn (c,n,a,f,r)				cmd16 scan N r time no data

General commands & LAM handling:

cam_init()					Open CAMAC channel

cam_exit()					Close CAMAC channel.

cam_inhibit_set( c )				ESONE controller type A N30A9F26

cam_inhibit_clear( c )				ESONE-A N30A9F24

cam_crate_clear( c )				ESONE-A N28a9F26

cam_crate_zinit( c )				ESONE-A N28A8F26 (Z for initialiZe)

or

cam_dataway_clear( c )				ESONE-A N28A9F26

cam_dataway_zinit( c )				ESONE-A N28A8F26 (Z for initialiZe)

cam_lam_enable(c,n)				enable LAM (CcNnA0F26)

cam_lam_disable(c,n)				disable LAM (CcNnA0F24)

cam_lam_read( c, &lam )			read encoded lam ESONE_A2 N30A0-7F0

cam_lam_clear( c, n)				clear encoded lam ESONE_A CcNnA0F10

External addresses functions:

came_cn(&ext,c,n,a)				bcna to ext composer

came_ext(ext,&,b,&c,&n,&a)			ext to bcna decomposer

cam16ei (ext,a,f,&d )				16 bit input

cam24ei (ext,a,f,&d )				32 bit input

cam16ei_q (ext,a,f,&d,&x,&q )			16 bit input with X and Q

cam24ei_q (ext,a,f,&d,&x,&q )			32 bit input with X and Q

cam16ei_r (ext,a,f,&d,r) 				16 bit input repeat until r

cam24ei_r (ext,a,f,&d,r) 				32 bit input repeat until r

cam16ei_rq (ext,a,f,&d,r) 			16 bit input repeat until noQ or max r

cam24ei_rq(ext,a,f,&d,r) 			32 bit input repeat until noQ or max r

cam16ei_saq (ext,a,f,&d,r) 			16 bit input scan A until noQ or max r

cam24ei_saq (ext,a,f,&d,r) 			32 bit input scan A until noQ or max r

cam16ei_snq (ext,a,f,&d,r) 			16 bit input scan A until noQ or max r

cam24ei_snq (ext,a,f,&d,r) 			32 bit input scan A until noQ or max r

cam16eo (ext,a,f,d)				16 bit output

cam24eo (ext,a,f,d)				32 bit output

cam16eo_q (ext,a,f,d,&x,&q)			16 bit output with X and Q

cam24eo_q (ext,a,f,d,&x,&q)			32 bit output with X and Q

camec (ext,a,f)					cmd16 without data

camec_q (ext,a,f,&x,&q)				cmd16 without data with X and Q

camec_sa (ext,a,f,r)				cmd16 scan A r time without data

camec_sn (ext,a,f,r)				cmd16 scan N r time without data

Interrupt handling:

cam_interrupt_enable ()				Enable interrupt

cam_interrupt_disable ()				Disable interrupt

cam_interrupt_attach ((func *)isr)		Attach interrupt func isr.

cam_interrupt_detach ()				Detach interrupt from ISR.

| KCS2926 CAMAC register structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Registers | Byte Addr. | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| CSR1 | +0 | Done | RFS | ABT | TMO | DERR | NOX | NOQ | GO |
| CSR2 | +1 | Done ENA | RFS ENA | RESET | 0 | 0 | ID1 | ID2 | BM |
| MCR | +2 | TM2 | TM1 | WS2 | WS1 | AD | RS4 | RS2 | RS1 |
| CAR | +3 | 0 | 0 | 0 | 0 | 0 | C4 | C2 | C1 |
| CR (NAF1) | +4 | A4 | A2 | A1 | F16 | F8 | F4 | F2 | F1 |
| CR (NAF2) | +5 | 0 | 0 | N16 | N8 | N4 | N2 | N1 | A8 |
| DR (DL) | +6 | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 |
| DR (DM) | +7 | d16 | d15 | d14 | d13 | d12 | d11 | d10 | d9 |
| DR (DH) | +8 | d24 | d23 | d22 | d21 | d20 | d19 | d18 | d17 |
| PP | +9 | C(7) | C(6) | C(5) | C(4) | C(3) | C(2) | C(1) | C(0) |

| HYTEC 1331 Turbo CAMAC register structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Registers | Addr. | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| Data low | +0 | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 |
| Data mid | +2 | d16 | d15 | d14 | d13 | d12 | d11 | d10 | d9 |
| Data high | +4 | d24 | d23 | d22 | d21 | d20 | d19 | d18 | d17 |
| read:status | +6 | X | SW1D | FAIL | Soft lam | Int. flag | Int. enb. | Inhibit | Q |
| write: A | +6 |  |  |  |  |  | A4 | A2 | A1 |
| read: lam | +8 |  |  |  | L16 | L8 | L4 | L2 | L1 |
| write: N | +8 |  |  |  | N16 | N8 | N4 | N2 | N1 |
| F | +10 |  |  |  | F16 | F8 | F4 | F2 | F1 |
| Crate #2 | +16 | ... |  |  |  |  |  |  |  |
| Crate #3 | +32 | ... |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |

| DSP6001/6002 CAMAC register structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Registers | Byte Addr | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| DWR (DH) | +0 | w24 | w23 | w22 | w21 | w20 | w19 | w18 | w17 |
| DWR (DM) | +1 | w16 | w15 | w14 | w13 | w12 | w11 | w10 | w9 |
| DWR (DL) | +2 | w8 | w7 | w6 | w5 | w4 | w3 | w2 | w1 |
| AR | +3 |  |  |  |  | A8 | A4 | A2 | A1 |
| FR | +4 |  |  |  | F16 | F8 | F4 | F2 | F1 |
| NR | +5 |  |  |  | N16 | N8 | N4 | N2 | N1 |
| ZCIR | +7. |  | AX4 | AX3 | AX2 | AX1 | I | C | Z |
| CACY | +8 | Any Value |  |  |  |  |  |  |  |
| LQX | +9 | L | L16 | L8 | L4 | L2 | L1 | X | Q |
| DRR (DH) | +10 | r24 | r23 | r22 | r21 | r20 | r19 | r18 | r17 |
| DRR (DM) | +11 | r16 | r15 | r14 | r13 | r12 | r11 | r10 | r9 |
| DRR (DL) | +12 | r8 | r7 | r6 | r5 | r4 | r3 | r2 | r1 |
| RSR | +13 |  |  |  |  |  | AO | AL | AC |

| KCS2927 CAMAC register structure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Registers | Byte Addr. | Bit15 | Bit14 | Bit13 | Bit12 | Bit11 | Bit10 | Bit9 | Bit8 | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| CSR | +0 | ERR | ABT | ITMO | RST IFC | 0 | PP | RFS | RFS IE | DNE | DNE IE | 0 | ID2 | ID1 | NoX | NoQ | Go |
| MCR | +2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | BLK | TM2 | TM1 | WS2 | WS1 | AD |
| CCR | +4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | C4 | C2 | C1 |
| NAF | +6 | RD NAF | 0 | N16 | N8 | N4 | N2 | N1 | A8 | A4 | A2 | A1 | F16 | F8 | F4 | F2 | F1 |
| DLR | +6 | d16 | d15 | d14 | d13 | d12 | d11 | d10 | d9 | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 |
| DHR | +8 |  |  |  |  |  |  |  |  | d24 | d23 | d22 | d21 | d20 | d19 | d18 | d17 |
| SSR | +8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | C7 | C6 | C5 | C4 | C3 | C2 | C1 | C0 |
| TCR | +10 | TC15 | TC14 | TC13 | TC12 | TC11 | TC10 | TC9 | TC8 | TC7 | TC6 | TC5 | TC4 | TC3 | TC2 | TC1 | TC0 |

# ESONE standard IEEE recommendations.

The subroutine (not function) names are composed of symbols which mean:

C : subroutine performing a Control function

D : subroutine performing a Declaration of a CAMAC entity

F : subroutine performing a transfer Full length (24 bit) data word

G : subroutine performing a analysis of a CAMAC entity into its address components

S : subroutine performing a transfer Short (less than 24bit) data words

T : subroutine performing a Test state of a signal or status indication

draft for esone.c:

PRIMARY subroutines:

cdreg (&ext, b,c,n,a)			Encode b,c,n,a into ext

cfsa (f, ext, &data , &q)			Action 24 bits

cssa (f, ext, &data, &q)			Action 16 bits

cccz (ext)				Z cycle (N28A8F26)

cccc (ext) 				C cycle (N28A9F26

ccci (ext, l)				Enable/Disable Inhibit

l=1 => N30A9F26

I=0 => N30A9F24

ctci (ext, &l)				Test the Inhibit

N30A9F27 Q=1 if I is set

cccd (ext, l)				Enable/disable Crate Demand

I = 1 => NnA0F26

I = 0 => NnA0F24

ctcd (ext, &l)				Test Crate demand enabled l = true or false

NnA0F8 Q=1 if I is set

ctgl (ext, &l)				Test crate demand present.

cdlam (&lam, b, c, n, m, inta[])		Declare LAM (like cdreg but for LAM) inta[] is

implementation-dependent.

m: mode of access of the LAM

m>=0 => subaddress, LAM access via dataless func

m<0 => negative of a bit position for a LAM

cclm (lam, l)				Enable/Disable LAM, F26 l=1, F24 l=0

cclc (lam)				Clear LAM F10

ctlm (lam, &l)				Test LAM F8

cclnk (lam, (*func)func)			Link LAM to Service procedure

BLOCK TRANSFERS, MULTIPLE ACTIONS and INVERSE DECLARATION:

cfga (fa[], exta[], intc[], qa[], cb[4])	General Multiple Action

cfmad (f, extb[2], intc[], cb[4])		Address scan

cfubc (f, ext, intc[], cb[4])		Controller-Synchronized block transfer

cfubl (f, ext, intc[], cb[4])			LAM-Synchronized block transfer

cfubr (f, ext, intc[], cb[4])			Repeat Mode block transfer.

cglam (lam, &b, &c, &n, &m, inta[])	Decompose LAM into b,c,n,m,inta.

cgreg (ext, &b, &c, &n, &a)		Decompose ext into b,c,n,a.